PROJECT 1: Detection of Heavy Metal Ions

| Project Title Detection of Heavy Metal Ions | Overview A portable paper-based colorimetric sensor designed to detect and estimate Ni²⁺ and Cr⁶⁺ ions in water. It combines capillary flow through Axiva paper with chemical indicators and computer-based RGB analysis. |
| --- | --- |
| Problem Industrial effluents can introduce toxic heavy-metal ions into water. Conventional spectroscopy and fluorescence methods can be precise but are bulky and expensive, creating a need for portable, low-cost field detection. | Solution Fabricate separate Cr⁶⁺ and Ni²⁺ detection zones on paper, react water samples with DPC and DMG, capture the resulting colours under controlled lighting, and use Python RGB analysis with calibration curves to estimate concentration. |

Novelty

The project combines a simple paper microfluidic sensor with image-based quantitative analysis. The blank zone helps account for lighting variation, while the RGB-to-concentration workflow converts a visual chemical response into a digital measurement without requiring bulky analytical instruments.

Tools & Technologies

Axiva filter paper 100R; SolidWorks for sensor layout and flow-path design.

Diphenylcarbazide (DPC) for Cr⁶⁺; Dimethylglyoxime (DMG) for Ni²⁺.

EDTA for reagent stabilization; K₂Cr₂O₇ and NiSO₄·6H₂O for standards.

Python for RGB extraction, calibration plots and concentration estimation.

Methodology

Select Axiva paper because its larger pore size provides better wicking than the considered Whatman Grade 42 paper.

Design channels and separate detection/blank zones; orient the main channel along the paper fibres.

Prepare standard Cr⁶⁺ and Ni²⁺ solutions through direct and serial dilution and apply them to the respective zones.

Record colour responses under consistent lighting, extract RGB intensities using Python, and correlate them with concentration through calibration curves.

Outcome / Key Result

Cr⁶⁺ was successfully detected with a consistent violet-to-red response and accurate RGB-based quantification. Ni²⁺ showed an unexpected violet response and inconsistent quantitative behaviour, so reliable visual detection was achieved but quantitative estimation remained limited.